SEL 703 – Internship Engineering

Ripudaman Thind

SmartHealth Band

A Wearable Sensor System with On-Device Analytics and Web Dashboard for Everyday and Health Monitoring

Overview:

This project develops a low-power wearable device that continuously monitors basic physiological and activity signals such as heart rate and steps, and transmits them to a web-based dashboard. Lightweight algorithms or ML models will run on the device and/or backend to derive higher-level health indicators (e.g., stress level, activity intensity, adherence to movement goals). The system is designed for both everyday users and patients, providing a generic but extendable platform for health and wellbeing monitoring.

Background / Problem

Instead of sporadic clinic visits, modern lifestyles and chronic health issues (such as cardiovascular risk, sedentary behavior, and stress) necessitate ongoing, inconspicuous monitoring. There are a lot of commercial wearables available, but they are frequently closed ecosystems, have limited customization options, and are difficult to incorporate into clinical or research processes. A versatile, open, and inexpensive wearable platform that can gather multi-modal physiological and activity data, apply lightweight algorithms or machine learning models, and display trends on a web dashboard for both general users and patients under supervision is required.

AIM

The objective of this project is to design, implement, and assess a microcontroller-based wearable health monitoring system that can reliably transmit the processed data to a cloud-connected web dashboard, perform on-device preprocessing and analytics, and track fundamental signals like heart rate, step count, movement, and basic derived metrics. In addition to being appropriate for use as a data source in clinical or research settings, the system should be sufficiently general for tracking daily wellness.

Block Diagram

[Sensors] → [Wearable Microcontroller + Edge Processing] → [Wireless Link] → [Gateway Device] → [Backend Server + Database + ML] → [Web Dashboard] → [User / Clinician]

Use-Cases

To meet practical industry demands, this project creates a wearable health monitoring system that combines sensing, embedded processing, data analytics, and web-based visualization. Through continuous physiological and activity data collecting, lightweight algorithmic or machine learning-based analysis, and secure data presentation via a web dashboard, the system is intended to assist both routine wellbeing monitoring and patient-oriented observation. The project clearly relates to the digital health, workplace wellness, and health technology sectors by showcasing the useful application of software engineering, IoT, and data analytics abilities in a professional setting.

Key Industries:

- Personal Health and Wellness: Tracking daily well-being and lifestyle awareness through heart rate, steps, and activity trends.

- Patient and Assisted Care Monitoring: Constant, non-invasive monitoring of physiological and activity patterns to provide clinicians and caregivers with insights based on trends rather than diagnoses.

- Workplace Health and Safety: Examining physiological load and activity levels to enhance efforts to prevent injuries and raise awareness of weariness.

- Research and Innovation: An open, expandable wearable platform that can be used for testing, data-driven research, and assessing machine learning models and embedded algorithms.

Components Researched

| Category | Component | Purpose in System | Available Options / Types | Why This Is Chosen / Notes |
| --- | --- | --- | --- | --- |
| Microcontroller | ESP32 / ESP32-S3 | Main controller; sensor data collection, processing, wireless communication | ESP32-WROOM-32, ESP32-S3 | Built-in Wi-Fi + BLE, sufficient RAM for edge ML, low cost, industry-standard for IoT [1] |
| Heart Rate Sensor (PPG) | MAX30102 / MAX30105 | Measures heart rate (and optional SpO₂) using photoplethysmography | MAX30102, MAX30105 | Widely used in wearables, accurate, compact, well-documented [2] |
| Motion Sensor | Accelerometer / IMU | Step counting, activity detection, movement analysis | MPU6050, LIS3DH, LSM6DS3 | Enables step detection and activity classification; LIS3DH is low-power |
| Temperature Sensor | Skin / Body Temperature Sensor | Tracks basic physiological temperature trends | MCP9808, MLX90614, NTC Thermistor | Adds health context; improves activity and stress interpretation |
| Power Source | Li-Po Battery (3.7V) | Portable power supply for wearable | 500mAh–1000mAh Li-Po | Rechargeable, compact, suitable for wearables |
| Battery Charging Module | Charging & Protection Circuit | Safe charging and battery protection | TP4056, MCP73831 | Prevents over-charging/discharging; essential for real device safety |
| Voltage Regulation | 3.3V Regulator (LDO) | Stable power for MCU and sensors | AMS1117-3.3, low-dropout LDOs | Ensures reliable sensor readings and MCU stability |
| Wireless Communication | BLE / Wi-Fi (on ESP32) | Data transmission to gateway/backend | BLE GATT, Wi-Fi TCP/HTTP | BLE for low power; Wi-Fi for simple demos and direct backend access |
| Display (Optional) | OLED Display | Real-time feedback (HR, steps, status) | 0.96” SSD1306 OLED | Improves usability and device realism |
| User Input | Push Button / Switch | Start/stop logging, user interaction | Tactile button, slide switch | Simple control for marking events or power |
| Haptic Feedback | Vibration Motor | Alerts and notifications | Coin vibration motor | Enhances user experience without screen dependency |
| Storage (Optional) | Local Data Storage | Offline data logging | MicroSD card module | Useful for testing, debugging, and data redundancy |
| Speech Input (Optional) | Microphone Module | Voice/speech capture (advanced feature) | INMP441 (I2S mic) | Enables speech-based psychological analysis (usually via gateway) |
| Gateway Device | Phone / PC / Raspberry Pi | Receives wearable data and forwards to backend | Android phone, Laptop, Raspberry Pi | Reduces wearable complexity; enables heavier ML |
| Backend Server | API + Database | Data storage, analytics, ML processing | Node.js / FastAPI, MongoDB | Scalable, industry-relevant backend architecture |
| Web Dashboard | Frontend Application | Data visualisation and monitoring | React, Next.js | Real-time charts, summaries, professional UI |
| Enclosure | Wearable Case | Protects electronics and allows wearing | 3D-printed case, enclosure box | Makes prototype a real wearable device |
| Strap / Mounting | Wrist / Body Mount | Attaches device to user | Watch strap, Velcro strap | Required for practical wearable testing |

[3]

Recent wearable IoT research, where ESP32 has been used to interface with biomedical sensors and transmit real-time health data in wearable health monitoring prototypes, supports the system's decision to use an ESP32 microcontroller as the central processing unit with integrated Wi-Fi/BLE. [4]


Research utilizing DS18B20 temperature sensors and MAX30102 photoplethysmography (PPG) sensors for heart rate monitoring shows their usefulness and dependability in comparable IoT wearable applications. 
[5]

Additionally, thorough analyses of wearable health systems contextualize the significance of combining several physiological sensors with microcontrollers to accomplish continuous health monitoring in applications targeted at both patients and consumers. 
[6]

For Alzheimer’s patients:

References

[1]	J. Landa, G. Barrios, and G. Huelsz, “IoT smartwatch based on open technologies for the collection of thermal comfort data,” HardwareX, vol. 22, p. e00633, Jun. 2025, doi: 10.1016/J.OHX.2025.E00633.

[2]	M. Pawar, S. Rusia, S. Tijare, A. Gadpalliwar, M. Ashtekar, and R. Pichkate, “Development of Health Monitoring Wearable Device using ESP32,” International Congress on Information and Communication Technology, pp. 2007–2012, 2024, doi: 10.1109/ICICT60155.2024.10544910.

[3]	“(PDF) Integrating Wearable Health Monitoring Devices with IoT for Enhanced Personal Health Management: A Comprehensive Review.” Accessed: Dec. 14, 2025. [Online]. Available: https://www.researchgate.net/publication/382878497_Integrating_Wearable_Health_Monitoring_Devices_with_IoT_for_Enhanced_Personal_Health_Management_A_Comprehensive_Review

[4]	“(PDF) Integrating Wearable Health Monitoring Devices with IoT for Enhanced Personal Health Management: A Comprehensive Review.” Accessed: Dec. 14, 2025. [Online]. Available: https://www.researchgate.net/publication/382878497_Integrating_Wearable_Health_Monitoring_Devices_with_IoT_for_Enhanced_Personal_Health_Management_A_Comprehensive_Review

[5]	“Ejournal.” [Online]. Available: www.circuitdigest.com

[6]	“(PDF) Integrating Wearable Health Monitoring Devices with IoT for Enhanced Personal Health Management: A Comprehensive Review.” Accessed: Dec. 14, 2025. [Online]. Available: https://www.researchgate.net/publication/382878497_Integrating_Wearable_Health_Monitoring_Devices_with_IoT_for_Enhanced_Personal_Health_Management_A_Comprehensive_Review

Week 4 Meeting Minutes:

Project Focus & Scope

- The project needs to be reduced to a single, distinct goal.

- Determine the wearable's main purpose:

  - Workplace Safety and Health (WHS)

  - or Patient/Health Monitoring

  - Don't create a wearable that can do everything.

- Concentrate on one main use case, mentioning the others simply as potential extensions.

Measurements & Data Collection

- Clearly state the metrics that the wearable will take

- The selected use case must be directly supported by the measurements.

- Examples of measurements (not all at once):

  - Heart rate

  - Movement and steps

  - Degree of activity

- No extraneous sensors unless they are essential to the main function.

Machine Learning & Algorithms

- ML ought to be:

  - Lightweight

  - Adequate for microcontroller operation

- To begin with:

  - A basic rule-based algorithm or machine learning model

  - Test performance first (feasibility, accuracy).

- At first, stay away from complicated models.

- ML isn't just utilized for complexity; it's also used when it adds value.

ML Development Approach

- Research:

  - Existing solutions

  - Similar studies

  - Practical machine learning techniques for edge devices

- Develop:

  - An initial ML model

  - Verify its functionality.

- Improve only if required after evaluation.

Overall, the supervisor emphasized during the discussion how crucial it is to further reduce the project's scope to keep it feasible and focused. A single principal use case, the precise measurements to be taken, and a constrained machine learning scope appropriate for microcontroller execution were all described. To prevent scope creep, the supervisor suggested outlining exactly what the system will accomplish and, conversely, what it will not try to address. It was emphasized that the final submission should be consistent with the original proposal, show iterative development from the original concept, and contain a critical assessment of design choices, trade-offs, and limits. Throughout development, professional decision-making, reflective practice, and problem-solving should be reflected in the project documentation and weekly logs.

`